SIMATS ENGINEERING (SAVEETHA SCHOOL OF ENGINEERING)

Department of Computer Science and Engineering

ASSESSMENT TOOL 2 – Short Answers

| Course Code: | ITA03 | Course Title: | Mobile computing |
| --- | --- | --- | --- |
| CO Assessed: | CO1 | Assessment: | Assessment Tool 2 – Short Answers |
| Weightage: | 30 % of CIA | Total Marks: | 100 Marks |
| CO1 Statement: | Analyze the mobile network components, mobile base station, mobile infrastructures and protocols in cellular systems. |  |  |
| Student Name: | V. Nagendra | Reg. No.: | 192425091 |
| Section / Batch: | AI & ML / 2024 | Date: |  |

Code of Conduct I am V. Nagendra (192425091) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary action. Signature of the Student: V. Nagendra

Instructions - This test contains 50 questions, Total: 100 Marks. - Duration: 1 hour.

| Faculty In-charge | Course Coordinator | Head of Department |
| --- | --- | --- |
| Signature: _______________ | Signature: _______________ | Signature: _______________ |
| Date: ___________________ | Date: ___________________ | Date: ___________________ |

Questions and Answers

Q1. A telecom operator wins a spectrum auction for 45 MHz. Company policy sets aside 25 kHz for every simplex channel. The planning team wants to know how many full-duplex voice channels this spectrum can support.

Sol: Simplex channels = total bandwidth / channel width = 45 MHz / 25 kHz = 1800

A full-duplex channel needs 2 simplex channels (forward + reverse)

Full-duplex channels = 1800 / 2 = 900

Q2. A regional carrier runs its network on 560 total voice channels, shared across a 7-cell frequency reuse cluster. During capacity planning, an engineer needs to know how many channels a single cell will typically be allocated.

Sol: Channels per cell = total channels / cluster size (N)

= 560 / 7

= 80 channels per cell

Q3. A GSM operator sets carrier spacing at 200 kHz and rolls out 100 carriers, each carrying 8 time slots for voice traffic. Determine the total number of simultaneous voice channels this deployment can offer.

Sol: Total channels = number of carriers × time slots per carrier

= 100 × 8

= 800 simultaneous voice channels

Q4. During a site audit, engineers note the base station radiates 80 W while a nearby handset transmits just 4 W. They want to express this power gap as a decibel-based power control ratio (10·log10 of the ratio).

Sol: Power ratio = 80 W / 4 W = 20

Ratio in dB = 10·log10(20)

= 13.01 dB ≈ 13 dB

Q5. A drive test captures a serving signal at -60 dBm alongside co-channel interference at -92 dBm. The RF team needs the Carrier-to-Interference ratio (C/I) in dB to assess link quality.

Sol: C/I (dB) = C (dBm) − I (dBm)

= −60 − (−92)

= 32 dB

Q6. An operator holds 42 MHz of spectrum, assigning 300 kHz per channel, and reuses frequencies across a 7-cell pattern. How many channels does a single cell end up with?

Sol: Total channels = 42 MHz / 300 kHz = 140

Channels per cell = total channels / N = 140 / 7

= 20 channels per cell

Q7. A transmitter delivers 64 W of power to a receiver. Engineers then move the receiver twice as far away, in an environment with a path-loss exponent of 4. What received power should they now expect?

Sol: Received power Pr ∝ 1 / d^n, with n = 4

Distance doubled, so Pr = 64 × (1/2)^4 = 64 / 16

= 4 W

Q8. Planners want to double a cell's radius while keeping the same SNR at the new edge, in an environment obeying an inverse-square power law. By what factor should transmit power be scaled up?

Sol: Inverse-square law: Pr ∝ 1 / d²

Radius doubled, so required power factor = 2² = 4

Transmit power must be increased 4 times

Q9. A network is built with a cluster size of 4, and every cell in the cluster is provisioned with 15 voice channels. What is the total channel count across the entire cluster?

Sol: Total channels in cluster = cluster size × channels per cell

= 4 × 15

= 60 channels

Q10. A drive test records a vehicle traveling at 100 km/h while camped on a 900 MHz carrier. Estimate the Doppler shift the receiver experiences.

Sol: v = 100 km/h = 27.78 m/s, λ = c / f = (3×10⁸) / (900×10⁶) = 0.333 m

Doppler shift fd = v / λ

= 27.78 / 0.333 = 83.3 Hz

Q11. A TDMA frame lasting 5 ms is divided into 8 equal time slots. What is the duration of a single slot?

Sol: Slot duration = frame duration / number of slots

= 5 ms / 8

= 0.625 ms (625 µs)

Q12. Spectrum planners have 24 MHz of allocated bandwidth and fix carrier spacing at 200 kHz for a GSM rollout. How many carrier frequencies can be created from this allocation?

Sol: Number of carriers = total bandwidth / carrier spacing

= 24 MHz / 200 kHz

= 120 carriers

Q13. A handset transmits at 8 W, but the signal experiences 33 dB of path loss on its way to the base station. Estimate the power the base station receiver actually sees, in dBm.

Sol: Transmit power = 10·log10(8 × 1000 mW) = 39.03 dBm

Received power = transmit power − path loss = 39.03 − 33

= 6.03 dBm

Q14. A network engineer uses 1 m as the reference distance while assessing a 1900 MHz deployment, where cell sites are spaced 2.5 km apart. Estimate the free-space path loss over that distance.

Sol: λ = c / f = (3×10⁸) / (1.9×10⁹) = 0.158 m, d = 2500 m

FSPL = 20·log10(4πd / λ) = 20·log10(4π × 2500 / 0.158) = 20·log10(1.99×10⁵)

= 106 dB (approximately)

Q15. A cluster consists of 6 cells, each able to carry up to 250 simultaneous calls at peak. What is the cluster's total call-handling capacity?

Sol: Cluster capacity = number of cells × calls per cell

= 6 × 250

= 1500 simultaneous calls

Q16. A switching subsystem is rated to process 4000 new call requests every second, and calls last 3 minutes on average. How many calls could be active in the system at any one time?

Sol: Holding time = 3 min = 180 s, arrival rate = 4000 calls/s

Active calls = arrival rate × holding time = 4000 × 180

= 720,000 calls

Q17. Engineers assign 150 channels to a base station subsystem, each channel occupying 25 kHz. What total bandwidth does this base station consume?

Sol: Total bandwidth = number of channels × channel width

= 150 × 25 kHz = 3750 kHz

= 3.75 MHz

Q18. Spectrum planners insert a 40 kHz guard band between each of 30 adjacent frequency bands to prevent interference. How much total bandwidth ends up wasted because of these guard bands?

Sol: Wasted bandwidth = number of guard bands × guard band width

= 30 × 40 kHz = 1200 kHz

= 1.2 MHz

Q19. A vehicle cruises at 90 km/h through a region of 1.5 km-radius cells. Using the cell diameter as the crossing distance, roughly how many handoffs would occur over an hour of travel?

Sol: Cell diameter = 2 × 1.5 km = 3 km, distance travelled in 1 hour = 90 km

Number of handoffs = distance travelled / cell diameter

= 90 / 3 = 30 handoffs

Q20. A carrier's network hierarchy has 8 regional centers, each overseeing 40 base stations, with each base station serving 250 users. What is the network's total user-handling capacity?

Sol: Total capacity = regional centers × base stations × users per base station

= 8 × 40 × 250

= 80,000 users

Q21. Coverage planners want to blanket a 400 sq km region using hexagonal cells of 400 m radius (approximate cell area as R²). Roughly how many cells will the rollout require?

Sol: Area of one cell ≈ R² = (0.4 km)² = 0.16 km²

Number of cells = total area / cell area = 400 / 0.16

= 2500 cells

Q22. An FDMA system allots 30 kHz to each channel, drawing from a total spectrum pool of 6 MHz. How many channels can be carved out of this allocation?

Sol: Number of channels = total spectrum / channel bandwidth

= 6 MHz / 30 kHz

= 200 channels

Q23. A market study notes that 1200 subscribers each place 4 calls per hour, averaging 2.5 minutes per call. What total offered traffic, in Erlangs, does this represent?

Sol: Traffic A = (subscribers × calls per hour × call duration in min) / 60

= (1200 × 4 × 2.5) / 60 = 12,000 / 60

= 200 Erlangs

Q24. A 2.5 GHz deployment has 40 MHz of bandwidth, and each connected user consumes 400 kHz. What is the maximum number of users the band can serve at once?

Sol: Maximum users = total bandwidth / bandwidth per user

= 40 MHz / 400 kHz

= 100 users

Q25. A carrier's base stations each cap out at 600 simultaneous users, and the network comprises 1200 such stations. What total user capacity does the network offer?

Sol: Total capacity = users per base station × number of base stations

= 600 × 1200

= 720,000 users

Q26. A paging scheme sweeps through 4 cells at a time across a 200-cell network. In the worst case, how many paging rounds are needed to locate a subscriber?

Sol: Paging rounds (worst case) = total cells / cells paged per round

= 200 / 4

= 50 rounds

Q27. A core network is engineered for 100,000 simultaneous connections. During a busy period it receives 40,000 call attempts per minute, each lasting 3 minutes on average. What fraction of the offered traffic ends up blocked?

Sol: Offered load = call attempts per min × duration = 40,000 × 3 = 120,000 calls

Blocked calls = 120,000 − 100,000 = 20,000

Fraction blocked = 20,000 / 120,000 = 0.1667 ≈ 16.7 %

Q28. A network policy lets 15% of its 600,000 subscribers roam onto partner networks each hour. At any given moment, how many subscribers are likely roaming?

Sol: Roaming subscribers = 15 % of total subscribers

= 0.15 × 600,000

= 90,000 subscribers

Q29. A switching center is capable of processing 12,000 calls per second, but is currently handling a peak load of 9,000 calls per second. What percentage of its capacity is being utilized?

Sol: Utilization = (current load / capacity) × 100

= (9,000 / 12,000) × 100

= 75 %

Q30. An operator reuses frequencies with a factor of 4 across a network holding 48 MHz of spectrum, with 200 kHz per channel. How many channels land in each cell?

Sol: Total channels = 48 MHz / 200 kHz = 240

Channels per cell = total channels / reuse factor = 240 / 4

= 60 channels per cell

Q31. A 900 MHz GSM deployment uses 200 kHz channel spacing across a 30 MHz allocation. How many channels can this spectrum support?

Sol: Number of channels = total allocation / channel spacing

= 30 MHz / 200 kHz

= 150 channels

Q32. Over the course of an hour, a network logs 2500 handoff attempts, 99% of which succeed. How many calls were successfully handed off?

Sol: Successful handoffs = success rate × attempts

= 0.99 × 2500

= 2475 handoffs

Q33. A mobile handset radiates 12 W, and the link experiences 45 dB of path loss before reaching the base station. What power level does the base station receiver detect?

Sol: Transmit power = 10·log10(12 × 1000 mW) = 40.79 dBm

Received power = 40.79 − 45

= −4.21 dBm (≈ 0.38 mW)

Q34. A 7-cell cluster is deployed where each cell can serve up to 900 simultaneous users. What is the cluster's overall user capacity?

Sol: Cluster capacity = number of cells × users per cell

= 7 × 900

= 6300 users

Q35. A TDMA carrier's frame spans 4.8 ms and is split into 8 equal slots. How long is each individual slot?

Sol: Slot duration = frame duration / number of slots

= 4.8 ms / 8

= 0.6 ms (600 µs)

Q36. A mobile device moves at 150 km/h while connected to a 1.8 GHz carrier. What Doppler shift should the receiver expect?

Sol: v = 150 km/h = 41.67 m/s, λ = c / f = (3×10⁸) / (1.8×10⁹) = 0.1667 m

Doppler shift fd = v / λ

= 41.67 / 0.1667 = 250 Hz

Q37. A core switch is dimensioned for 250,000 simultaneous calls. It is fielding calls at a rate of 70,000 per minute, each averaging 4 minutes in duration. What share of the offered load gets blocked?

Sol: Offered load = 70,000 × 4 = 280,000 calls

Blocked calls = 280,000 − 250,000 = 30,000

Fraction blocked = 30,000 / 280,000 = 0.107 ≈ 10.7 %

Q38. A hexagonal layout groups 9 cells into a cluster, and each cell can host 600 users. What is the cluster's combined user capacity?

Sol: Cluster capacity = cells per cluster × users per cell

= 9 × 600

= 5400 users

Q39. A switching center processes 60,000 calls per second, with 0.4% of them dropped due to network issues. How many calls are being dropped every second?

Sol: Dropped calls per second = drop rate × calls per second

= 0.004 × 60,000

= 240 calls per second

Q40. A base station in an 1800 MHz network transmits at 43 dBm. At the edge of its 2 km-radius cell, the received signal measures -87 dBm. What path loss has the signal undergone?

Sol: Path loss = transmitted power − received power (in dBm)

= 43 − (−87)

= 130 dB

Q41. A single base station covers 2.5 km² of territory, and the operator plans to deploy 150 identical stations. What total geographic area will the rollout cover?

Sol: Total area = area per station × number of stations

= 2.5 km² × 150

= 375 km²

Q42. An operator applies a frequency reuse factor of 6 over a 48 MHz spectrum allocation, with each channel taking up 200 kHz. How many channels does each cell get?

Sol: Total channels = 48 MHz / 200 kHz = 240

Channels per cell = total channels / reuse factor = 240 / 6

= 40 channels per cell

Q43. A paging system processes 6000 users per cycle, and the subscriber base totals 150,000 users. How many paging cycles are needed to sweep the entire base?

Sol: Paging cycles = total subscribers / users per cycle

= 150,000 / 6000

= 25 cycles

Q44. A phone call runs for 9 minutes, and during that time the network hands the call off roughly every 3 minutes. How many handoffs took place over the call?

Sol: Number of handoffs = call duration / handoff interval

= 9 min / 3 min

= 3 handoffs

Q45. A receiver picks up a wanted signal at -85 dBm against a noise floor of -115 dBm. What signal-to-noise ratio, in dB, does this represent?

Sol: SNR (dB) = signal (dBm) − noise floor (dBm)

= −85 − (−115)

= 30 dB

Q46. A 4G cell is serving 250 users simultaneously, each needing a guaranteed 150 kbps. What is the minimum bandwidth the cell must provide to support them all?

Sol: Minimum bandwidth = users × rate per user

= 250 × 150 kbps = 37,500 kbps

= 37.5 Mbps

Q47. A base station handles 1500 subscribers, each averaging 4 calls per hour with a 2.5-minute call length. What total traffic, in Erlangs, does this generate?

Sol: Traffic A = (subscribers × calls per hour × call duration in min) / 60

= (1500 × 4 × 2.5) / 60 = 15,000 / 60

= 250 Erlangs

Q48. A network reserves 250 separate control channels, each 12 kHz wide, for signaling. What total bandwidth is tied up in control channels?

Sol: Total bandwidth = number of control channels × channel width

= 250 × 12 kHz = 3000 kHz

= 3 MHz

Q49. A network is carrying 1500 simultaneous connections, each streaming at 600 kbps on average. What total throughput must the network support?

Sol: Total throughput = connections × rate per connection

= 1500 × 600 kbps = 900,000 kbps

= 900 Mbps

Q50. A telecom operator wants each tower to cover a 4 km radius (treating coverage as a circle, area = πR²) and needs to blanket a 350 km² service region. Roughly how many towers will the rollout require?

Sol: Area covered per tower = πR² = π × 4² = 50.27 km²

Number of towers = total area / area per tower = 350 / 50.27 = 6.96

≈ 7 towers